| MARINE SPECIFICATION |  |  |
| --- | --- | --- |
| LENGTH | m | 55.168 |
| MAX LENGTH | m | 56.61 |
| BREADTH | m | 28.041 |
| DEPTH | m | 4.876 |
| JACKHOUSE | m | 6.4 main deck to house top. |
| DRAFT | m | 3.32 summer, actual add 1.6 to tip of spud can. |
| MAX VDL MOVING | Te | 227 |
| MAX VDL ELEVATED | Te | 227 |
| NAVIGATION |  | Simrad Robertson |
|  |  | AP9Mk2 Autopilot |
|  |  | ROBPOS RDP600 joystick control. |
|  |  | CD109 course detector. |
| PROPULSION CONFIG. |  | 4 azimuthing thrusters 2 on bow, 2 on stern 3928 kW |
|  |  | 8 kt |
| MAIN PROPULSION |  | Aquamaster |
| drive type | - | Caterpillar 3512 |
| props type | - | US 1401 / 3000 |
| power | kW | 2 x 1119 kW |
| other |  |  |
| THRUSTERS |  |  |
| Mnftr |  | Aquamaster |
| Model |  | US 1401 / 3000 |
| drive type |  | Elect var Hz |
| power |  | 2 x 895 kW |
| other |  |  |
| MAIN POWERPLANT |  |  |
| Engines | qty | 3 |
| Mnftr | - | Caterpillar |
| Model | - | 3512 |
| total power | kW/VAC | 3210 / 600 |
| EMERGENCY GENERATOR | kW | 250 kw / 480 V |
|  |  | Caterpillar 3406 |
| WINCHES | each | 2 |
| Type | - | single drum |
| rated pull | Te | 5.5 |
| holding cap | Te | 8 |
| speed | m/min | 6 |
| drum capacity | m | 300 |
| cable size | mm | 50 |
| ANCHORS | each | 2 |
| Weight | Te | 1.5 |
| Type | - |  |
| LEGS | each | 4 |
| Type | - | LeTourneau latice |
| Size | m | 3.97 triangle |
| Length | m | 73.24 |
| footing area / leg | m2 | 23 |
| spuds retractable |  | semi 1.6m pr’tuson |
| jetting system |  | in cans only |
| JACKING SYSTEM |  |  |
| Type | - | Rack & pinion |
| Speed | m / hr | 219.6 raise & lower legs |
|  |  | 109.8 raise hull |
|  |  | Siemens var Hz |
| FUEL STORAGE | m3 | 208 |
| POTABLE WATER | m3 | 300 |
| INDUSTRIAL WATER | m3 | 230 (In Brine Tanks) |
| ACCOMODATION |  |  |
| permanent beds | beds | 46 |
| 1 man cabins | - | 6 |
| 2 man cabins |  | 18 |
| 4 man cabins |  | 0 |
| deck modules |  | can fit if required |
| air conditioned |  | TR yes |
| galley seats |  | 22 |
| LIFEBOATS |  |  |
| Nr |  | 2 |
| Model |  | Watercraft 8.5 Mk IV |
| Type |  | Gravity davit |
| Capacity |  | 60 |
| total capacity |  | 2 x 60 |
| LIFERAFTS | each | 6 RFD |
| total capacity |  | 6 x 20 |
| HELIDECK |  |  |
| helicopter type |  | Dauphine 365N |
| refuelling facilities |  | No |